LOGSTASH

- Logstash is an open source tool designed to manage all of your server logs in a centralized location.

- The servers that are running Logstash agents are called shippers. They send log events of your applications and services to the Logstash server.
- The central Logstash server running services such as brokers, indexers, and storage interface with Kibana, which is a visualization tool.

- Logstash collects data from the different sources defined by using the configuration file.

-  Elasticsearch does real-time data analysis from different data sources.

- Kibana is a web application designed to visualize data in Elasticsearch.

SETUP ENVIRONMENT

1. Install Java

sudo yum install java-1.8.0-openjdk

2. Install LogStash

a. Download and install the public signing key

rpm --import https://artifacts.elastic.co/GPG-KEY-elasticsearch

b. Create a new file /etc/yum.repos.d/logstash.rep

[logstash-6.x]

name=Elastic repository for 6.x packages

baseurl=https://artifacts.elastic.co/packages/6.x/yum

gpgcheck=1

gpgkey=https://artifacts.elastic.co/GPG-KEY-elasticsearch

enabled=1

autorefresh=1

type=rpm-md

c. Run sudo yum install logstash

Note : logstash will be installed etc/share/logstash

Start and check statu service of logstash :

systemctl enable logstash.service

systemctl start logstash.service

systemctl status logstash.service

RUN CONFIGURE FILE :

./logstash -f /etc/logstash/conf.d/config.conf

3. Install Elasticsearch

a. Do as 2.a

b. Create a new file /etc/yum.repos.d/elasticsearch.rep

[elasticsearch-6.x]

name=Elasticsearch repository for 6.x packages

baseurl=https://artifacts.elastic.co/packages/6.x/yum

gpgcheck=1

gpgkey=https://artifacts.elastic.co/GPG-KEY-elasticsearch

enabled=1

autorefresh=1

type=rpm-md

[sudo systemctl enable elasticsearch.service], [sudo chkconfig --add elasticsearch],

[sudo /bin/systemctl daemon-reload

sudo /bin/systemctl enable elasticsearch.service]

- : will add Elasticsearch to init scripts, so
that it will start while CentOS  is booting

Elasticsearch can be started and stopped using the service command:

- sudo -i service elasticsearch start

- sudo -i service elasticsearch stop

c. Test Server

- Uncomment network.host and http.port in file /etc/elasticsearch/ elasticsearch.yml

- Restart server : sudo service elasticsearch restart

-  To test our installation to see if it is running on port 9200

C1. Run : curl -X GET http://huydu02-e20652:9200/

C2. Go  http://huydu02-e20652:9200/ to test server

4. Install Kabian

a. Do 2.a or 3.a

b. Create a new file /etc/yum.repos.d/kibana.rep

[kibana-6.x]

name=Kibana repository for 6.x packages

baseurl=https://artifacts.elastic.co/packages/6.x/yum

gpgcheck=1

gpgkey=https://artifacts.elastic.co/GPG-KEY-elasticsearch

enabled=1

autorefresh=1

type=rpm-md

Run : sudo yum install kibana

Use the chkconfig command to configure Kibana to start automatically when the system boots up:

sudo chkconfig --add kibana

You can start and stop Kibana using the service command:

sudo -i service kibana start

sudo -i service kibana stop

The output that you see is the most common output that we generally see
from Logstash:
• "message" includes the complete input message or the event line
• "@timestamp" will include the timestamp of the time when the event was
indexed; or if date filter is used, this value can also use one of the fields in the
message to get a timestamp specific to the event
• "host" will generally represent the machine where this event was generated

Check config before run : bin/logstash –configtest ../conf/logstash.conf